Using the ModelSim-Intel FPGA Simulator with Verilog Testbenches

3 files included in "Modelsim" subfolder to control Modelsim: testbench.v, testbench.tcl, and wave.do

- testbench.v: The purpose of a testbench is to instantiate a Verilog module that is to be simulated, and to specify values for its inputs at various simulation times

timescale:

1ns  Unit simulation time (so that #20 is 20ns)

1ps  Granularity of time that ModelSim evaluates the signal (evaluation precision)

- wave.do: Specify the signals (waves) to be shown

- testbench.tcl: Specifies ModelSim commands to run simulation

- Memory is instantiated by inst_mem.mif

We need to have a memory module (inst_mem.v) in our code, which is connected to inst_mem.mif

testbench.tcl compile .v files, call "vsim …"           testbench.v: Simulate signals and call our module

call "do wave.do"  Configure the waves of the signals being produced

DESim Software

Runs projects

- addern.v is the file that contains the module to be tested

- top.v is the top-level module that connects the devices (in DESim) with the module - consistent with other projects

- sim folder:

"Compile testbench" button in DESim will execute the run_compile.bat file

+) Remove (if already exist) a work folder to store results of simulation

+) Compile tb.v (testbench.v) inside tb folder

+) Compile Addern.v and top.v

tb.v  Calls top.v  Calls Addern.v

"Start simulation" will execute the run_sim.bat file.

Simulate project with memory module

Inside the run_compile file:

- Line 1-3 copies inst_mem.mif to sim folder

+) ModelSim requires the file to be in the sim folder to properly initialize the memory module during a simulation

+) If the file is changed in the display folder, then the latest version of the file will always be used when starting a simulation

- Line 4-6 deletes the file that sometimes causes an error to ModelSim

- You should not need to make any changes to the files in the sim or tb folder for your new project in my_folder.

You can now open your new project in the DESim software and proceed to compile/simulate your code.

"Compile testbench"  run_compile.bat  Copy .mif to sim & create work folder  (Sets up memory)

Compiles tb.v: Setup devices (HEX0-6, LEDR0-9, VGA, etc.) Calls top module

Compiles top.v: Connect needed devices to proc module

Compiles proc.v: Our module - Calls inst_mem module

Compiles inst_mem.v: called by proc

"Start simulation"  run_sim.bat